Packet Tracer Practical Lab – Network Configuration & Troubleshooting

Scenario: Small Office Network Setup

You are a Network Technician at TechCore Solutions. The company is setting up a small office network, but the current configuration is incomplete and not fully functional.

Your task is to configure the network and resolve connectivity issues so all devices can communicate properly.

Network Topology (to be built in Packet Tracer)

Devices:

- 1 Router (R1)

- 1 Switch (S1)

- 3 PCs (PC1, PC2, PC3)

- 1 Server

Connections:

- PCs and Server → Switch

- Switch → Router

Requirements

IP Addressing Scheme

| Device | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| R1 (G0/0) | 192.168.10.1 | 255.255.255.0 | — |
| Server | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC1 | DHCP | — | — |
| PC2 | DHCP | — | — |
| PC3 | DHCP | — | — |

Tasks

Task 1 – Basic Network Setup

- Connect all devices correctly.

- Configure the router interface with the given IP address.

- Assign a static IP to the server.

Task 2 – DHCP Configuration

- Configure the router (R1) as a DHCP server.

- Create a DHCP pool:

  - Network: 192.168.10.0/24

  - Default gateway: 192.168.10.1

  - DNS server: 192.168.10.10

- Exclude IP addresses:

  - 192.168.10.1 – 192.168.10.20

Task 3 – Troubleshooting

After configuration, PC2 cannot access the server.

Students must:

- Test connectivity using:

  - ping

  - ipconfig

- Identify the issue.

- Fix the problem.

Task 4 – Verification

Ensure that:

- All PCs receive IP addresses from DHCP

- All PCs can ping:

  - Router

  - Server

  - Each other

Task 5 – Documentation

Provide:

- Screenshot of IP configuration for one PC

- Screenshot of DHCP pool configuration

- Short explanation of the issue found and how it was fixed

Skills Assessed

- Router configuration (basic CLI)

- DHCP setup

- IP addressing

- Network troubleshooting

- Verification using networking tools

Optional Challenge (Advanced)

- Configure a second subnet (192.168.20.0/24)

- Add another router interface and test routing

- Implement basic access control (ACL)

Instructor Tip (Hidden Issue Idea)

(Do NOT give to students)

You can introduce one of these:

- Wrong subnet mask on PC2

- Incorrect default gateway

- DHCP pool misconfigured

- Interface shutdown on router